# This file is a general .xdc for the Basys3 rev B board

# To use it in a project:

# - uncomment the lines corresponding to used pins

# - rename the used ports (in each line, after get_ports) according to the top level signal names in the project

# Clock signal

# set_property PACKAGE_PIN W5 [get_ports clk]

# set_property IOSTANDARD LVCMOS33 [get_ports clk]

# create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk]

# Switches

# set_property PACKAGE_PIN V17 [get_ports {sw[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[0]}]

# set_property PACKAGE_PIN V16 [get_ports {sw[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[1]}]

# set_property PACKAGE_PIN W16 [get_ports {sw[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[2]}]

# set_property PACKAGE_PIN W17 [get_ports {sw[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[3]}]

# set_property PACKAGE_PIN W15 [get_ports {sw[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[4]}]

# set_property PACKAGE_PIN V15 [get_ports {sw[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[5]}]

# set_property PACKAGE_PIN W14 [get_ports {sw[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[6]}]

# set_property PACKAGE_PIN W13 [get_ports {sw[7]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[7]}]

# set_property PACKAGE_PIN V2 [get_ports {sw[8]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[8]}]

# set_property PACKAGE_PIN T3 [get_ports {sw[9]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[9]}]

# set_property PACKAGE_PIN T2 [get_ports {sw[10]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[10]}]

# set_property PACKAGE_PIN R3 [get_ports {sw[11]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[11]}]

# set_property PACKAGE_PIN W2 [get_ports {sw[12]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[12]}]

# set_property PACKAGE_PIN U1 [get_ports {sw[13]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[13]}]

# set_property PACKAGE_PIN T1 [get_ports {sw[14]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[14]}]

# set_property PACKAGE_PIN R2 [get_ports {sw[15]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {sw[15]}]

# LEDs

# set_property PACKAGE_PIN U16 [get_ports {led[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[0]}]

# set_property PACKAGE_PIN E19 [get_ports {led[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[1]}]

# set_property PACKAGE_PIN U19 [get_ports {led[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[2]}]

# set_property PACKAGE_PIN V19 [get_ports {led[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[3]}]

# set_property PACKAGE_PIN W18 [get_ports {led[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[4]}]

# set_property PACKAGE_PIN U15 [get_ports {led[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[5]}]

# set_property PACKAGE_PIN U14 [get_ports {led[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[6]}]

# set_property PACKAGE_PIN V14 [get_ports {led[7]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[7]}]

# set_property PACKAGE_PIN V13 [get_ports {led[8]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[8]}]

# set_property PACKAGE_PIN V3 [get_ports {led[9]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[9]}]

# set_property PACKAGE_PIN W3 [get_ports {led[10]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[10]}]

# set_property PACKAGE_PIN U3 [get_ports {led[11]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[11]}]

# set_property PACKAGE_PIN P3 [get_ports {led[12]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[12]}]

# set_property PACKAGE_PIN N3 [get_ports {led[13]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[13]}]

# set_property PACKAGE_PIN P1 [get_ports {led[14]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[14]}]

# set_property PACKAGE_PIN L1 [get_ports {led[15]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {led[15]}]

# 7 segment display

# set_property PACKAGE_PIN W7 [get_ports {seg[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}]

# set_property PACKAGE_PIN W6 [get_ports {seg[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}]

# set_property PACKAGE_PIN U8 [get_ports {seg[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}]

# set_property PACKAGE_PIN V8 [get_ports {seg[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}]

# set_property PACKAGE_PIN U5 [get_ports {seg[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}]

# set_property PACKAGE_PIN V5 [get_ports {seg[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}]

# set_property PACKAGE_PIN U7 [get_ports {seg[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}]

# set_property PACKAGE_PIN V7 [get_ports dp]

# set_property IOSTANDARD LVCMOS33 [get_ports dp]

# set_property PACKAGE_PIN U2 [get_ports {an[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]

# set_property PACKAGE_PIN U4 [get_ports {an[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

# set_property PACKAGE_PIN V4 [get_ports {an[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

# set_property PACKAGE_PIN W4 [get_ports {an[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

# Buttons

# set_property PACKAGE_PIN U18 [get_ports btnC]

# set_property IOSTANDARD LVCMOS33 [get_ports btnC]

# set_property PACKAGE_PIN T18 [get_ports btnU]

# set_property IOSTANDARD LVCMOS33 [get_ports btnU]

# set_property PACKAGE_PIN W19 [get_ports btnL]

# set_property IOSTANDARD LVCMOS33 [get_ports btnL]

# set_property PACKAGE_PIN T17 [get_ports btnR]

# set_property IOSTANDARD LVCMOS33 [get_ports btnR]

# set_property PACKAGE_PIN U17 [get_ports btnD]

# set_property IOSTANDARD LVCMOS33 [get_ports btnD]

# Pmod Header JA

# set_property PACKAGE_PIN J1 [get_ports {JA[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[0]}]

# set_property PACKAGE_PIN L2 [get_ports {JA[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[1]}]

# set_property PACKAGE_PIN J2 [get_ports {JA[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[2]}]

# set_property PACKAGE_PIN G2 [get_ports {JA[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[3]}]

# set_property PACKAGE_PIN H1 [get_ports {JA[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[4]}]

# set_property PACKAGE_PIN K2 [get_ports {JA[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[5]}]

# set_property PACKAGE_PIN H2 [get_ports {JA[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[6]}]

# set_property PACKAGE_PIN G3 [get_ports {JA[7]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JA[7]}]

# Pmod Header JB

# set_property PACKAGE_PIN A14 [get_ports {JB[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[0]}]

# set_property PACKAGE_PIN A16 [get_ports {JB[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[1]}]

# set_property PACKAGE_PIN B15 [get_ports {JB[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[2]}]

# set_property PACKAGE_PIN B16 [get_ports {JB[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[3]}]

# set_property PACKAGE_PIN A15 [get_ports {JB[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[4]}]

# set_property PACKAGE_PIN A17 [get_ports {JB[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[5]}]

# set_property PACKAGE_PIN C15 [get_ports {JB[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[6]}]

# set_property PACKAGE_PIN C16 [get_ports {JB[7]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JB[7]}]

# Pmod Header JC

# set_property PACKAGE_PIN K17 [get_ports {JC[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[0]}]

# set_property PACKAGE_PIN M18 [get_ports {JC[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[1]}]

# set_property PACKAGE_PIN N17 [get_ports {JC[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[2]}]

# set_property PACKAGE_PIN P18 [get_ports {JC[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[3]}]

# set_property PACKAGE_PIN L17 [get_ports {JC[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[4]}]

# set_property PACKAGE_PIN M19 [get_ports {JC[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[5]}]

# set_property PACKAGE_PIN P17 [get_ports {JC[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[6]}]

# set_property PACKAGE_PIN R18 [get_ports {JC[7]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JC[7]}]

# Pmod Header JXADC

# set_property PACKAGE_PIN J3 [get_ports {JXADC[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[0]}]

# set_property PACKAGE_PIN L3 [get_ports {JXADC[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[1]}]

# set_property PACKAGE_PIN M2 [get_ports {JXADC[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[2]}]

# set_property PACKAGE_PIN N2 [get_ports {JXADC[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[3]}]

# set_property PACKAGE_PIN K3 [get_ports {JXADC[4]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[4]}]

# set_property PACKAGE_PIN M3 [get_ports {JXADC[5]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[5]}]

# set_property PACKAGE_PIN M1 [get_ports {JXADC[6]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[6]}]

# set_property PACKAGE_PIN N1 [get_ports {JXADC[7]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[7]}]

# VGA Connector

# set_property PACKAGE_PIN G19 [get_ports {vgaRed[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[0]}]

# set_property PACKAGE_PIN H19 [get_ports {vgaRed[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[1]}]

# set_property PACKAGE_PIN J19 [get_ports {vgaRed[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[2]}]

# set_property PACKAGE_PIN N19 [get_ports {vgaRed[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[3]}]

# set_property PACKAGE_PIN N18 [get_ports {vgaBlue[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[0]}]

# set_property PACKAGE_PIN L18 [get_ports {vgaBlue[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[1]}]

# set_property PACKAGE_PIN K18 [get_ports {vgaBlue[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[2]}]

# set_property PACKAGE_PIN J18 [get_ports {vgaBlue[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[3]}]

# set_property PACKAGE_PIN J17 [get_ports {vgaGreen[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[0]}]

# set_property PACKAGE_PIN H17 [get_ports {vgaGreen[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[1]}]

# set_property PACKAGE_PIN G17 [get_ports {vgaGreen[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[2]}]

# set_property PACKAGE_PIN D17 [get_ports {vgaGreen[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[3]}]

# set_property PACKAGE_PIN P19 [get_ports Hsync]

# set_property IOSTANDARD LVCMOS33 [get_ports Hsync]

# set_property PACKAGE_PIN R19 [get_ports Vsync]

# set_property IOSTANDARD LVCMOS33 [get_ports Vsync]

# USB-RS232 Interface

# set_property PACKAGE_PIN B18 [get_ports RsRx]

# set_property IOSTANDARD LVCMOS33 [get_ports RsRx]

# set_property PACKAGE_PIN A18 [get_ports RsTx]

# set_property IOSTANDARD LVCMOS33 [get_ports RsTx]

# USB HID (PS/2)

# set_property PACKAGE_PIN C17 [get_ports PS2Clk]

# set_property IOSTANDARD LVCMOS33 [get_ports PS2Clk]

# set_property PULLUP true [get_ports PS2Clk]

# set_property PACKAGE_PIN B17 [get_ports PS2Data]

# set_property IOSTANDARD LVCMOS33 [get_ports PS2Data]

# set_property PULLUP true [get_ports PS2Data]

# Quad SPI Flash

# Note: CCLK_0 cannot be placed in 7 series devices. Use STARTUPE2 primitive.

# set_property PACKAGE_PIN D18 [get_ports {QspiDB[0]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[0]}]

# set_property PACKAGE_PIN D19 [get_ports {QspiDB[1]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[1]}]

# set_property PACKAGE_PIN G18 [get_ports {QspiDB[2]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[2]}]

# set_property PACKAGE_PIN F18 [get_ports {QspiDB[3]}]

# set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[3]}]

# set_property PACKAGE_PIN K19 [get_ports QspiCSn]

# set_property IOSTANDARD LVCMOS33 [get_ports QspiCSn]